| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 1 | Low voltage during system start | F, H, L, F-AC, H-AC | Low voltage ELECTRICAL SYSTEM | Check battery voltage, chargebattery; Checkwireconnectiontocontroller; check main current fuse; check main contactor; replace control part; Replacecomponent; (Limit VN = 80V: VBatt < 64V or VCap. < 56V Limit VN = 48V: VBatt < 34V or VCap. < 34V Limit VN = 24V: VBatt <17V) | Travel is inhibited. Reduced power for travel and lifting. Warning symbol flashing (Else-Check). |
| 2 | Overvoltage | F, H, L, F-AC, H-AC | Overvoltage BATTERY CONTROLLER | Check battery voltage; replace controllerpart; replace component; (Limit VN = 80V: VBatt > 92 V Limit VN = 48V: VBatt > 62 V Limit VN = 24V: VBatt > 28 V) | No travel or hydraulic functions Warning symbol flashing (Else-Check). |
| 3 | Temperature limit for controllers > 80°C (>70°C MP1510C) or ACS only: Info at > 85°C Err at > 115°C Err at < -20°C | F, H, L, F-AC, H-AC | TEMPERATURE CONTROLLER | Check sensor and wire connections Allow controllers to cool down; check fan operation, improve cooling. Warm up the controller and operate at reduced level until the controller has warmed up. | Reduced power for travel and lifting. Warning symbol flashing (Else-Check). |
| 4 | Emergency Disconnect applied during travel | F | POWER SUPPLY | Warning (V < 20V MP1510C) | Brake applies, travel inhibited |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 5 | Overvoltage | F, F-AC, H-AC | ELECTRICAL SYSTEM | Check wire connection to battery, checkBattery, replacecontroller part; Replace component; (Limit for Retrak: VBatt > 62 V Limit for MP1510C: V > 32 V) | Brake applies, travel inhibited |
| 6 | Wire breakage: - Travel setpoint device | C, I2, I3, I4, I5, Travel switch | TRAVEL SWITCH | Check wire connection; Replace travel setpoint device; Replace Canion/ Interface; | Travel stop. “Warning” symbol flashes (Else-Check). Hydraulic functions possible |
| 7 | Key switch applied duringtravel | F,F-AC,H-AC | KEY SWITCH | Warning (V < 13V for 200ms MP1510C) | Brake applies, travel inhibited |
| 8 | 2 travel directions applied simultaneously | F-AC, H-AC, C, MP, S, I2, I3, I5, FS | ELECTRICAL SYSTEM | Warning, replace travel direction switch, replace Interface 7 Canion; replace Multipilot / Solopilot For twin pedal, check the electrical system. | Travel stop. “Warning” symbol flashing (Else-Check). Hydraulic functions possible |
| 9 | Operator protection switch faulty | I2, FS | ELECTRICAL SYSTEM | Check switch, check wire connections |  |
| 10 | V Cap. <> V Batt; (main contactor jammed or not open) | F, H, F-AC, H-AC L | MAIN CONTACTOR | Check main contactor, check wire connection; check output pin 2 (ACS) of traction controller; replace component | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |
| 11 | Offset error armature / field current (DC) Stator current (AC); Current measurement point | F, H, L, F-AC, H-AC | CONTROLLER | Check main current wire, replace component | Travel no function |
| 12 | Disable driver field rheostat faulty for DC Disable driver power stage faulty (forAC) | F, H F-AC, H-AC L | CONTROLLER | Replace component; for AC: Check flat belt wire connection; replace power part | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 13 | Disable Watchdog field faulty | F, H | CONTROLLER | Replace components | Travel no function |
| 14 | - Main contactor notclosed - Field power stage cannot be switched on (DC) | F, H, F-AC, H-AC | CONTROLLER/ MAIN CONTACTOR | Check wire connection; check main contactor; check charging connection, Check field supply fuse, Replace component. | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |
| 15 | Disable field faulty; Illegal address (for AC) | F, H, F-AC, H-AC L | CONTROLLER | Replace controlpart; replace component | Travel no function |
| 16 | Disable driver armature actuator faulty; Software reset (for AC) | F, H, F-AC, H-AC | CONTROLLER | Replace controlpart (AC); replace component | Travel no function |
| 17 | Disable Watchdog armature faulty | F, H | CONTROLLER | Replace components | Travel no function |
| 18 | DC: Armature power stage cannot be switched on AC: one of the 3 phases cannot be switched on | F, H, F-AC, H-AC L | CONTROLLER | Check armature wire connection; replace power part; replace component | Travel no function |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 19 | Disable armature faulty; Power stage short circuit (for AC) The error may be triggered by the faulty speed sensor in connection with AS4814Plus controllers. Sensor wheel wobbles (elliptical) or the sensor- sensor wheel gap is too big. In this case the component is not faulty. A short circuit cannot be resolved by switching the truck off/on. | F, H, F-AC ,H- AC, L-AC | CONTROLLER | Replace power part; Replace component. Can occur in connection with error 70, in this case: check power supply behind the main contactor contact, in the event of an error the voltage will be much lower here than the rated operating voltage. Disconnect the positive supply from the components to target the error cause. The exception here is the F1 traction controller which must remain connected. One component overloads the testing voltage due to internal short circuit. The testing voltage arises by charging the internal capacitor batteries via a resistor to the level of the control voltage supply. | Travel no function |
|  |  |  |  | Check sensor, replace sensor and / or sensor wheel. |  |
| 20 | Armature incorrectly connected | F, H | ELECTRICAL SYSTEM | Check armature main current wire connection | Travel no function |
| 21 | Armature overvoltage (short circuit / high side faulty) | F, H, F-AC, H-AC L | CONTROLLER/ ELECTR | Check main current wire to armature; check settings(for AC); AArmature > 360A for 250ms (MP1510); Replace component. |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 22 | Watchdog error; Watchdog does not run out | F, H, L, M, S, A, I2, I3, I4, C, F-AC, H-AC, FS | CONTROLLER/ CANION | Truck reset, Replace controlpart; replace component | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |
| 23 | - Computer overload - Permanent interrupt - “ACS” timeout on CAN connection | F, H, L, M, S, A, I2, I3, I4, C, F-AC, H-AC, FS | CONTROLLER / CAN | Check CAN bus connection; Check CANION connector and component; Replace component | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |
| 24 | EEPROM cannot be contacted / parameters not plausible / parameter set to default Faulty checksum | F, H, L, I2, I3, I4, C, F-AC, H-AC MFC brake MFC hydr. , travel switch | CONTROLLER/ CANION | Check battery cable; check VBatt; set trucktype, set default parameters; replace control part; replace component Delete logbook, change parameters, set hourmeter, replace component | Flashing warning symbol (Else- Check). |
| 25 | No telegrams: - no logon from interface - no telegrams from Can-Bus travel switch | F, H, I4, I8 F-AC, H-AC L | INTERFACE / CAN | Check CANBus connection, check wire connection between proportional valve – modulation; check fuse 1F8; replace iInterface (I2, I4, I8) ; replace lift potentiometer, Multipilot, Solopilot or interface; MP1510C: Voltage collapses, controller logs on again, interface does not, controller remains blocked and waits for interface. Check battery voltage (with load!); charge or replace battery if necessary; AS2409K: no “live” telegram received from CAN Bus travel switch for more than 300 ms; replace travel switch |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 26 | Wire breakage: - Lift potentiometer | M, S, I2, I3 | MULTIPILOT / SOLOPILOT / LIFT POTENTIOMETER | Check wire connection | Lift stop. “Warning” symbol flashes (Else-Check), travel enabled |
| 27 | Wire breakage: - Steering setpoint device; - Reach truck: for 45° steering bolster movement no flank from proximity switch. - Canion: “Steer angle acquire active” parameter has incorrect value - Juniors PM2402L: “Operating mode” parameter incorrectly set | C, L, I3, I4 | STEER ACTUAL ANGLE | Check wire connection; Replace steer angle sensor / potentiometer; Replace Canion / Interface; Replace steering; PM2402L from software version 1.60: set parameter P511 “operating mode” correctly  1 = analog sensor (potentiometer), 2 = digital sensor on steer motor | Crawl speed, Flashing warning symbol (Else- Check), hydraulic functions enabled. This error produces an emergency stop on the reach truck and the Juniors. |
| 28 | Wire breakage: - Steering sensor | C, L, I3, I4 | STEER NOMINAL ANGLE | Check wire connection, replace steering wheel sensor;replace Canion / interface; replace steering | Crawl speed, Flashing warning symbol (Else- Check), hydraulic functions enabled. This error produces an emergency stop on the reach truck |
| 29 | Wire breakage: - Traverse potentiometer | M | Traverse nominal value MULTIPILOT | Replace Multipilot |  |
| 30 | Wire breakage: - Tilt potentiometer | M | Tilt nominal value MULTIPILOT | Replace Multipilot, Teach Multipilot (Hall sensor 2003 only) |  |
| 31 | Wire breakage: - ZH1 setpoint device - Initial lift / lower Setpoint device | M, S Travel switch | Sideshift nominal value MULTIPILOT / SOLOPILOT | Replace Multipilot / Solopilot, Teach Multipilot (Hall sensor 2003 only); Travel switch: Check / replace switch | Hydraulic functions stop, “Warning” symbol flashes (Else-Check), travel enabled |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 32 | Wire breakage: - ZH2 setpoint device | M, S | MULTIPILOT / SOLOPILOT | Replace Multipilot / Solopilot, Teach Multipilot (Hall sensor 2003 only) | Hydraulic functions stop, “Warning” symbol flashes (Else-Check), travel enabled |
| 33 | Wire breakage: - ZH3 setpoint device | M, I8 | MULTIPILOT | Replace Multipilot |  |
| 34 | CANBus damaged | F, H, L, M, S, A, I2, I3, I4, I5, C, F- AC, H-AC Travel switch | CAN-BUS | Check CAN bus connection; replace faulty component; Check terminal resistances | No travel, hydraulics Warning symbol Flashing warning (Else-Check). Only stored in logbook (from 2004). |
| 35 | No travel zero position Deadman and accelerator pedal applied when truck switched on | I2, I3, I4, C Travel switch | TRAVEL ZERO POSITION | Warning, check travel switch; Intermittent contact in interface power supply (12 volts) Check Canion, for twin pedal check electrical system | Travel stop, Flashing warning symbol (Else- Check), hydraulic functions enabled. |
| 36 | No lift zero position | M, I2, I3,FS | LIFT ZERO POSITION | Warning, check lift setpoint device. | Hydraulic functions stop, “Warning” symbol flashes (Else-Check), travel enabled |
| 37 | No steering angle nominal value | L | CONTROLLER / CAN | Check steering setpoint device;Check CANBus connection; check “steer type” parameter; Replace component (I5) |  |
| 38 | Component does not log on, self test not concluded. | F, H, L, I2, I3, I4, C, A, M, S, F-AC, H-AC | CONTROLLER / CAN | Check CAN-Bus connection and voltage; check DIL switch 7 encoding (MP1510); steering controller monitoring activated? (AS2412F); check ACS encoding; replace component | No travel, lifting Warning symbol Flashing warning (Else-Check). |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 39 | Implausible truck type 1. Discovered during the self test phase that at least one component has a different truck type as a parameter. A different truck type was discovered in operating mode. Caused by intermittent contacts in the power supply connections | F, H, L, I2, I3, I4, I5, A, M, F-AC, H-AC | TRUCK TYPE | For 1. Set truck type; For 2. Examine and if necessary replace switch (key switch) or rectify intermittent contact in power supply plug connections. Set truck type. |  |
| 40 | Motor overtemperature Traction motor overtemperature in operating mode.> 150°C And travel speed < 1km/h For ACS: Info > 145°C Err > 165°C | L, F, H F-AC, H-AC | TEMPERATURE MOTOR | Warning; check motor sensor system and wire connections; Check motor; traction motor brake open? | Reduced power for travel and lifting. Warning symbol flashing (Else-Check). For steering only warning symbol flashing. |
| 41 | Brake magnet potentiometer Potentiometer voltage less than 0.3 volts or greater than 2.7 volts for more than 500ms. | L | BRAKE MAGNETPOTENTIOM ETER | Check potentiometer; check potentiometer attachment; check wire connection |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 42 | - Short circuit between | L,F | BRAKE MAGNET | Check wire connection, check brake |  |
|  | the output and battery |  |  | magnet; replace component |  |
|  | positive.(EJC, ELE) |  |  |  |  |
|  | -Brake magnet control, |  |  |  |  |
|  | below minimum difference |  |  |  |  |
|  | on potentiometer for 500 |  |  |  |  |
|  | ms when magnet applied. |  |  |  |  |
|  | (150mV ) |  |  |  |  |
|  | No brake pressure during |  |  |  |  |
|  | initialization. |  |  |  |  |
|  | Brake pressure after the |  |  |  |  |
|  | Emergency Stop magnet |  |  |  |  |
|  | has applied; |  |  |  |  |
| 43 | Adjust brake | L | ADJUST BRAKE | Adjust brake; check potentiometer |  |
| 44 | -Brake not connected or connection wire short circuit -Brake cable torn Only for steer category_3: Internal driver of failsafe brake cannot connect the relay Or short circuit bypass (contact jams) on the output during power up test | L,F | BRAKE FAULTY | -Check brake wiring, replace brake if necessary -Replace brake cable, check potentiometer; Check potentiometer attachment; Only for steer category_3: -Check brake driver output, check switch contact plausibility, Replace steer component. | No release |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 45 | Too many pulses from steering setpointdevice Intermittent contact or perturbing radiation. More than 255 pulses identified within 100ms | L,I5 | STEERING WHEEL | Check wire connection, replace sensor bearing; replace component |  |
| 46 | Steering setpoint devicewrongly connected AS4803L: More than 3 pulses identified in one direction and more than 3 pulses in another within 100ms. | L,I5 | STEERING WHEEL | Check sensor bearing connection; check wireconnectio AS4803L: Check steering wheel recognition input. |  |
| 47 | No pulses from steer | L,I5 | STEERING WHEEL | Check wire connection, replace sensor | No release |
|  | setpointdevice |  |  | bearing; replace component |  |
|  | ETVQ steering wheel no |  |  |  |  |
|  | pulses from 1st sensor |  |  |  |  |
|  | bearing. |  |  |  |  |
|  | More than 10 pulses were |  |  |  |  |
|  | obtained from the stepper |  |  |  |  |
|  | motor but fewer than 3 |  |  |  |  |
|  | pulses were counted |  |  |  |  |
|  | within 200ms from the |  |  |  |  |
|  | sensor bearing. |  |  |  |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 48 | Does not match the "Steering type" parameter ETVQ steering wheel no pulses from stepper motor or 2nd sensor bearing. More than 10 pulses were obtained from the sensor bearing but fewer than 2 pulses were counted within 200ms from the stepper motor sensor bearing. | L | STEERING TYPE | Set steering type correctly ETVQ: Check stepper motor coupling, tighten grub screw. Test both setpoint device branches, set defaults and truck type Otherwise faulty, replace component. |  |
| 49 | CAN supply interrupted; Supply above 14 volts or less than 10 volts, Can or sensor bearing incorrectly supplied | AS 2412 F,H-AC | Can Bus CAN supply | Suply on CAN connector greater than 14 volts (electrical system short circuit, component) | Travel disabled |
| 50 | NRG Multipilot wire breakage Horizontal tilt | M | Setpoint device horizontal | Check Multipilot wire connection Parameter 540 set to 1 by mistake | Travel and lifting disabled. |
| 51 | Teach-In: steering actual value potentiometer | L | Incorrect setting SETTING | Repeat calibration, check wire connection;check potentiometer;replace component |  |
| 52 | Teach-In: Brake magnet potentiometer | L | Incorrect setting SETTING | Repeat calibration, check wire connection;check potentiometer;replace component |  |
| 53 | Teach-In: Steering setpoint potentiometer | L | Incorrect setting SETTING | Repeat calibration, check wire connection;check potentiometer;replace component |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 54 | Motor wire breakage during power-up test ETVQ: Temperature voltage level less than 0.3 volts or greater than 2.7 volts | L, F, H | MOTOR WIRE BREAKAGE | Check wire connection; Check temperature sensor lines; replace component; replace motor |  |
| 55 | Motor short circuit; Current not ok; Power stage faulty | L, F-AC, H-AC | SHORT CIRCUIT MOTOR | Check wire connection; Replace motor; Replace component | Traction motor stops, Hydraulic motor stops Warning symbol flashing (Else-Check). |
| 56 | Motor or actual value transmitter incorrectly connected | L,F.H | MOTOR REVERSE POLARITY | Check motor connection; check actual value transmitter potentiometer; AC: Phases L1,L2 or L3 swapped on motor; |  |
| 57 | No actual value change Motor turns despite control signal | L,F,H | Steering controller ACTUAL VALUE TRANSMITTER | Check motor; check actual angle potentiometer; rectify mechanical wheel block; AS4803L: Check incremental bearings; replace component; AC: Check actual value transmitter wire breakage signals PM2402L check operating mode |  |
| 58 | Steering angle actual value fluctuates ETVQ: Reference mark 0° could not be started within 5 seconds or no valid segment reached. Steer motor does not turn | L | STEER ACTUAL ANGLE | Check actual value transmitter (on steer motor) Check steer motor, fuse and incremental bearing, rectify mechanical wheel block; Check proximity switch distance. |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 59 | Steering angle nominal value fluctuates | L | Steer angle setpoint device TILLER POTENTIOMETER | Check setpoint device potentiometer (on tiller) |  |
| 60 | Swivelling bolster does not follow steering wheel directions Deviation of > 15° for 100ms The wheel turns more than 5°without change of nominal value on steering wheel, less than 2°/sec after reaching the nominal position. Tyres over-tensioned, the actual value cannot follow the nominal value. | L | STEERING CONTROL | Check steering bolster mechanics; check chain and chain tension; check motor; check actual value transmitter attachment; Check tyres, replace wheel if necessary. |  |
| 61 | Power stage error | L | CONTROLLER | Replace controller |  |
| 62 | Power stage error | L | CONTROLLER | Replace controller |  |
| 63 | Sensor power supply out of range AS4803L: < 10V oder >16V | L,F-AC,H-AC | CONTROLLER | The 12 volt supply not in valid range. Monitoring of 12 volt sensor supply greater than 15 volts and less than 11 volts. Switch controller off and on, replace fuse and controller if necessary. |  |
| 64 | Software version differs | L | CONTROLLER | Different software versions in the AS4803Ls in the ETV-Q. Update software to same level. |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 65 | NRG Multipilot wire breakage centre shifter | M | Centre switch MP centre shifter | Check Multipilot wire connection Parameter 540 set to 1 by mistake | Travel and lifting disabled. |
| 66 | NRG Multipilot wire breakage 180° / 360° steering toggle | M | 180-360 switch MP toggle switch 180°/ 360° | Check Multipilot wire connection Parameter 543 set to 1 by mistake | Travel and lifting disabled. |
| 67 | NRG-Multipilot setpoint device travel direction switch jams on MP when system starts | M | Travel direction MP setpoint device | Check Multipilot switch, values in Judit MP teach window, recalibrate values | Travel and lifting disabled. |
| 68 | Safety switch wire breakage | I2, I3, I4, I5 | DEADMAN | Check wire connection to safety switch (deadman, seat switch); replace safety switch |  |
| 69 | Both lift and lower requested simultaneously | I2, I5, FS | LIFT/LOWERSWITCH | Check wire connection; check switch |  |
| 70 | Component fails to respond orresponds incorrectly to self test | F-AC, H-AC, M, S, A, I2, C | SELF TEST | Check displays on basic component (LED); note other error messages on basic card; Check wireconnectionsandcheck connector; check power supply behind the main contactor contact, in the event of an error the voltage will be much lower here than the rated operating voltage. Disconnect the positive supply from the components to target the error cause. One component overloads the testing voltage due to internal short circuit. The testing voltage arises by charging the internal capacitor batteries via a resistor to the level of the control voltage supply. | No travel, hydraulics Warning symbol Flashing warning (Else-Check). |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 71 | Lower potentiometer wire breakage | I2, FS | LOWER POTENTIOMETER | Check wire connection; replace lower potentiometer; replace Multipilot; replace interface |  |
| 72 | No lowering zero position | I2, FS | LOWER RESTPOSITION | Warning, check lower setpoint device |  |
| 73 | Valve output interface faulty | I2 | VALVE OUTPUT | Warning, check valve outputs |  |
| 74 | Interface valve output faulty | I2 | INTERFACE FAULTY | Replace interface |  |
| 75 | Proportional valve does not draw any current Correct Multipilot ?? | I2,I4,C | PROP. VALVE | Check wire connection, replace proportional valve;replace Canion / interface; replace proportional valve | Flashing warning symbol (Else- Check). |
| 76 | Proportional valve short circuit | I2, I4C | PROP. VALVE | Check wire connection, replace proportional valve;replace Canion / interface; replace proportional valve | Flashing warning symbol (Else- Check). |
| 77 | Button wire breakage | I2, | BUTTON WIRE BREAKAGE | Check wire connection; replace button; replace interface |  |
| 78 | Button and travel switch pressed simultaneously | I2 | BUTTON / TRAVEL SWITCH | Warning: Check wire connection; replace button; replace interface |  |
| 79 | Switch and deadman switch pressed simultaneously | I2 | BUTTON / DEADMAN | Warning: Check wire connection; replace button; replace interface |  |
| 80 | 2 travel directions simultaneously | I2 | ELECTRICAL SYSTEM | Check wire connection; replace button; replace interface |  |
| 81 | Different parameters for the two traction controllers | F, F-AC | MOTORS | reset parameters (EFG traction controllers) | Travel inhibited |
| 82 | Sensor wire breakage | I2 | ELECTRICAL SYSTEM | Safety height: Check wire connection; replace button; replace interface |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 83 | 2 of the same components report on thetruck e.g. AS4803L: Key switch voltage cannot be assigned to components Elle_1,Elle_2 or Elle_3 | F, H, I4, F-AC L-AC | ELECTRICAL SYSTEM | Check component encoding; check wire connection; replace component; | Travel inhibited |
| 84 | Faulty speed sensor: - AS4803L: - Speed changes by 50% within 4ms; - Reference speed less than half the rated speed; - Steer transmission blocked; - Tear identification: a valid sensor reading is reduced to zero within 4ms and remains at this level for 50ms without producing a new valid reading. - PM2402L/2 from V1.60: - Too many pulses on encoder 3 on steer motor | F,H,L-AC | SPEED SENSOR | Check speed sensor; Sensor bearing drift; Check wire connection; Replace speed sensor; Replace component; Check wire routing, do not route together with motor supply wires! | Travel stop, Lift stop, Flashing warning symbol (Else- Check), hydraulic functions enabled. |
| 85 | Power release low voltage Travel/Steer: Capacitor voltage drops to 30V (48V) for 0.5 seconds | F,H-AC L-AC | ELECTRICAL SYSTEM TRAVEL | Switch truck on and off; check battery; check battery cable connections; check fuse; Check power connections; replace component | Travel, hydraulics no function, warning symbol Flashing warning (Else-Check). |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 86 | Improbable speed, sudden change: - AS4803L: 2 sensor systems differ from each other. - PM2402L/2 from V1.60: Encoders 3 and 4 supply different sensor signalse | F,H-AC L-AC | SPEED IMPLAUS | Check wire connection; Replace speed sensor; Replace component; Pulse counts correct: 8 or 64 pulses/ revolution? Check wiring and plug connections, check wire routing, do not route together with motor supply wires! |  |
| 87 | Sensor line interrupted AS4803L: no voltage drop on a 100 Ohm internal measurement resistor MFC brake: load wheel speed sensing | F-AC L-AC MFC brake | Electrics DISCONNECTION SENSOR BEARING | Sensor wire disconnected, motor consumes maximum current and makes buzzing noise. Check sensor wires. |  |
| 88 | Hydraulic speed sensor faulty on AS4814H | H (AS4814H) | SPEED SENSOR H | Check speed sensor; Check wire connection; replace speed sensor; replace component; | Hydraulics stop, Flashing warning symbol (Else- Check), travel enabled. |
| 89 | Hydraulics power release low voltage | H | ELECTRICS- HYDRAULICS | Check battery, check battery wire connection; Check fuse; replace component | Reduced power for travel and lifting. Warning symbol flashing (Else-Check). |
| 90 | Travel with handbrake on | C | ELECTRICAL SYSTEM | Check magnetic brake switch | Flashing warning symbol (Else- Check). |
| 91 | 24 volts out of range Info at <17 V Err at <15 V | C | ELECTRICAL SYSTEM | Check power supply; check wire connection | For a warning, only the warning symbol flashes (Else-Check). For an error, the truck stops. |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 92 | Wrong component reporting on Canion | M, S, A | ELECTRICAL SYSTEM | Check Master/Solopilot; check display; replace component Check jumper in XB 27 and XB 44 from pin 2 to pin 6, | Travel, lifting function, warning symbol flashes (Else-Check). |
| 93 | ACS capacitors not charged | F, H, C | ELECTRICAL SYSTEM | Check charging connection, main contactor does not apply, check bus voltage, check transformer, fuses and cable set, Replace component | Travel, lifting no function, warning symbol flashes (Else-Check). |
| 94 | SDO error | F, H, C | SDO ERROR | Software incompatible check software version Reload software. Check jumper in XB 41 from pin 12 to pin 13. | Travel, lifting no function, warning symbol flashes (Else-Check). |
| 95 | Pin code error | C | PIN CODE ERROR | Component pin codes not identical | Travel, lifting no function, warning symbol flashes (Else-Check). |
| 96 | No zero position for hydraulics Masterpilot, Solopilot, | M,S | Hydraulics zero position MP/SOLOPILOT | Check Master/Solopilot; lift, lower, tilt, ZH1, ZH2, ZH3, horn switch not in zero position Replace component | Hydraulic functions stop, “Warning” symbol flashes (Else-Check), travel enabled |
| 97 | Segment beyond tolerance limits - PM2402L/2 from V1.60: Referencing error on power up, no switch signal from proximity switch (NS) or time interval exceeded | L | FAULTY SEGMENT | Segment on swivelling bolster not within tolerance level. Check wiring and proximity switch, check distance from switch to trip cam on swivelling bolster |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 98 | The position of the sensor bearing and proximity switch (segment) deviate by more than 1.1 degrees. When the truck starts up, the direction of rotation from the steering wheel sensor system to the motor power connection or the steer motor sensor system is inversed. - PM2402L/2 from V1.60: Referencing error during operation, unexpected switch signal from proximity switch. | L | POSITION DEVIATION | Check wiring. Check wiring and proximity switch, check distance from switch to trip cam on swivelling bolster |  |
| 99 | CANBus damaged; CANBus driver temporarily deactivated | F,H,M,S,A, I3, I4, C | CAN-BUS | Check wire connection, check terminal resistors, install truck anti- interferencemeasures; | Travel, hydraulics no function, warning symbol flashes (Else-Check). |
| 100 | NRG software versions of internal component parts implausible | M | Multipilot software | Replace component part with corresponding component, as flashing currently not possible in the field. |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 101 | Input / output error | JUBES | Jubes input/output | Check I/Oa, rectify contact error, check operating voltage, replace external transmitter Index info in display: a--- Sensor 1 supplies values, but not defined. Check parameters 11-17 b--- Sensor 2 supplies values, but not defined. Check parameters 20-26 c--- Sensor 3 supplies values, but not defined. Check parameters 29-35 Or a sensor 1-3 supplies a voltage outside the range of 0.5V – 9.5V. | Shown in the driver’s display with error 101 |
| 102 | Internal error | JUBES | Jubes internal | Check operating voltage. Index info in display: a --- Communication between access module and data recording faulty, check connection e --- Radio module faulty f --- Radio channel number is “0”, should be >0, check parameter 100 g --- Radio transmitting power is “0”, Check parameter 101 h --- Lock number is invalid, replace access module i --- Impact sensor faulty, replace data recorder | Shown in the driver’s display with error 102 |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 103 | Parameter error | JUBES | Jubes parameters | Set parameters Index info in display: a --- Parameter 5 greater than parameter 6, should be smaller. b --- Differential value of parameter 11 is greater than the differential values of the various stages of parameters 12-17 c --- Differential value of parameter 20 is greater than the differential values of the various stages of parameters 21-26 d --- Differential value of parameter 29 is greater than the differential values of the various stages of parameters 30-35 e --- the stages of parameters 12-17 … f --- the stages of parameters 21-26 … g --- the stages of parameters 30-35 … h --- Impact stages of parameters 47-49 … i --- Impact stages of parameters 50-52 … …are not in increasing order | Shown in the driver’s display with error 103 |
| 104 | Sensor bearing provides no pulses on system start up | F,H | (SENSOR BEARING) | Check sensor bearing and wire, replace sensor bearing or controller |  |
| 105 | Motor temperature gauge provides incorrect values Sensor line wire breakage | F,H,L | Temp.Motor | Check sensor connection and lines, if possible replace sensor, otherwise replace the entire motor… |  |
| 106 | Controller temperature gauge provides incorrect values | F,H,L | Temp. controller | Internal error, replace controller. |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 107 | Key switch voltage out of range Hardware encoding incorrect | F,H,L MFC hydr. MFC brake | Electrics | Error in controller connection (Saab connector) on component | No travel No lifting No steering Main contactor open |
| 108 | Calibration reading incorrect | F,H | (CONTROLLER internal) | Internal error, replace controller. |  |
| 109 | Brake pedal faulty | MFC brake | BRAKE PEDAL | Brake pedal signals implausible |  |
| 110 | Current output faulty | MFC brake MFC hydr. MFC option | CONTROLLER | Check outputs for short circuits | MFC_Hydr.: no lifting MFC_Brake: crawl speed only |
| 111 | Steering controller is monitored by traction controller for signs of life telegrams Traction controller does not receive these telegrams. The Can Bus is overloaded. | F | Steering controller | Check steering controller | Travel inhibited Main contactor open |
| 112 | Steering controller_2 is monitored by traction controller_1 for signs of life telegrams The Can Bus is overloaded. | F | Traction controller | Check Can Bus, measure terminal resistor 60Ohm |  |
| 113 | Lift controller does not send lift telegrams | H | LIFT CONTROLLER |  |  |
| 114 | No traverse distance reference on power up | MFC hydr. | Info 32 Traverse reference | None | Reduced traverse speed until both stops have been reached once. |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 115 | Main contactor improbable, contacts not closed. Wire connection interrupted, faulty main contactor driver on traction controller | F,H,L ?? | Main contactor improbable | Check wire connection. If error 19 is also present, disconnect the shorted power stage Disconnect the power positive. Check driver voltage (switched against negative). If reading OK, check contactor coil and mechanics. | No truck functions |
| 116 | After software flashing (field operation), the item number of the software is implausible and parameters may also be implausible. | F,H, MFC brake MFC prop | Info 54 SET TRUCK TYPE | Set truck type and switch truck off / on (wait approx. 35 seconds). On request, automatically: all parameters are set to default, the software item number is set to plausible, the log book is deleted, the hourmeter set to zero and entry 116 made in the log book. | In the log book only error 116 stands as a marker for this process and all parameters can be at default. |
| 117 | Cab position implausible, rotary potentiometer value does not match 180 degree switch | CANION | Cab position | Check rotary potentiometer, re-adjust if necessary, check 180 degree switch | No TRAVEL release |
| 118 | Steering controller reports error | L | Steering controller | Check display on steering controller | TRAVEL inhibited, EMERGENCY STOP activated |
| 119 | Weigher button faulty | MFC brake | Button faulty | Check wire and button | Weighing interrupted |
| 120 | Weigher pressure sensor faulty | MFC hydraulics | Sensor faulty | Check wire and sensor | Weighing interrupted |
| 121 | Battery incorrectly locked | F-AC | Battery lock | Locking strips stuck. Check proximity switch | No truck functions |
| 201 | Optical sensor faulty | Rack height select | sensor |  |  |

| Error | Description | Component | Display text | Cause / Action | Effect |
| --- | --- | --- | --- | --- | --- |
| 202 | Too many reference points during calibration | Rack height select | Calibration wrong |  |  |
| 203 | Sensor evaluation improbable | Rack height select | sensor |  |  |
| 204 | Error when reading EEPROM | Rack height select | Controller |  | All settings set to default |
| 205 | No Multipilot in truck | Rack height select | MP missing |  |  |
| 240 | Internal process interrupted | Rack height select | Controller |  |  |
| 250 | No height actual value on job input | Rack height select |  |  | The height is invalid for the warehouse level entered |
| 252 | Rack height select incorrect | Rack height select |  |  |  |
| 253 | Rotary field implausible – transmission overloaded, excess load on incline | F | Controller | None, switch truck off/on | Main contactor opened |
| 254 | Diagnostics piratry | Judit Box | None | Create authorisation | No diagnosis possible |

# Components:

| A | C = 064 | Display / Operation (LISA / JULIA) |
| --- | --- | --- |
|  | C = 065 | Display / Operation (Else-Check) |
|  | C = 066 | Display/Operation (CANCode) |
|  | C = 067 | Display (CANDis) |
|  | C = 068 | Display/Operation (on board computer) |
|  | C = 3 | Comfort display [CANopen] |
| C |  | Canion (Atlas Copco) [CANopen] |
|  | C = 1 | - Master |
|  | C = 11 | - Slave |
| F | C = 016 | Travel (MP1510C / AS2405F / AS2409K / AS2412F) |
|  | C = 016 | Travel 1 (MP1514F / AS4814Fplus) |
|  | C = 017 | Travel 2 (MP1514F / AS4814Fplus) |
| F-AC | C = 016 | Travel (AS4812/14F) |
|  | C = 8 | ACS Traction (Atlas Copco AC 8011/18, Inmotion) [CANopen] |
| Travel switch | C = 096 | Can-Bus travel switch |
| H | C = 032 | Lift (MP1514H / AS4814Hplus) |
| H-AC | C = 032 | Lift (AS4814H) |
|  | C = 7 | ACS Pump (Atlas Copco AC 8011/18, Inmotion) [CANopen] |
| Rack height select | C = 128 | Rack Select |
| I | C = 21 | I/O-Board 1 [CANopen] |
|  | C = 22 | I/O-Board 2 [CANopen] |
|  | C = 23 | I/O-Board 3 [CANopen] |
|  | C = 24 | I/O-Board 4 [CANopen] |

| I2 | C = 146 | Interface 2 |
| --- | --- | --- |
|  | C = 155 | Interface 21 |
|  | C = 156 | Interface 71 |
| I3 | C = 147 | Interface 3 |
| I4 | C = 148 | Interface 4 = MFC_brake |
| I5 | C = 149 | Interface 5 |
| I6 | C = 150 | Interface 6 = MFC_Prop |
| I7 | C = 151 | Interface 7 |
| I8 | C = 152 | Interface 8 = MFC_Option |
| Jubes | C = 069 | ISM Access Module |
|  | C = 29 | ISM Access Module [CANopen] |
|  | C = 070 | ISM-Data recorder |
|  | C = 071 | ISM-radio module |
| L | C = 048 | Steer (MP1502L / PM2402L / AS4803L) |
|  | C = 048 | Steer 1 (AS4803L) |
|  | C = 049 | Steer 2 (AS4803L) |
|  | C = 050 | Steer 3 (AS4803L) |
|  | C = 4 | Steer computer [CANopen] |
| M | C = 096 | Multipilot / Masterpilot |
|  | C = 2 | Multipilot / Solopilot [CANopen] |
| S | C = 112 | Solopilot / Hand lever |

Travel setpoint device: travel potentiometer, accelerator pedal

Steering actual value transmitter: steer potentiometer, proximity switch (on steered wheel)

# ONLY in 1st generation!

All messages numbered above 100 come from the interface and correspond to messages 1- xx -99!

| Error | Description/Cause | Component | Display text | Action | Effect |
| --- | --- | --- | --- | --- | --- |

| Warning / Information Messages |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | INFO 01 | SEAT SWITCH_OPEN |  |  |
|  |  | INFO 02 | NO_TRAVEL DIRECTION |  |  |
|  |  | INFO 03 | NO_TRAVEL RELEASE |  |  |
|  |  | INFO 04 | NO_NOMINAL VALUE |  |  |
|  |  | INFO 05 | LIFTING OFF |  |  |
|  |  | INFO 06 | DEADMAN KEY OPEN |  |  |
|  |  | INFO 07 | START_INCORRECT |  |  |
|  |  | INFO 08 | PARKING BRAKE |  |  |
|  |  | INFO 09 | SLOW SPEED |  |  |
|  |  | INFO 10 | TEMPERATURE DRIVE MOTOR |  |  |
|  |  | INFO 11 | TEMP_LIFT_MOTOR |  |  |
|  |  | INFO 12 | TEMP_STEER_MOTOR |  |  |
|  |  | INFO 13 | BRUSH WEAR OUT_DRIVE_MOTOR |  |  |
|  |  | INFO 14 | BRUSH WEAR OUT_LIFT_MOTOR |  |  |
|  |  | INFO 15 | BRUSH WEAR OUT_STEER_MOTOR |  |  |
|  |  | INFO 16 | DRIVING DISABLE |  |  |
|  |  | INFO 17 | LIFT END DIS. |  |  |
|  |  | INFO 18 | SECURITY HEIGHT |  |  |
|  |  | INFO 19 | DOOR OPEN |  |  |
|  |  | INFO 20 | AISLE MODE |  |  |
|  |  | INFO 21 | RESCUE STEERING |  |  |
|  |  | INFO 22 | STEERING MISSING |  |  |
|  |  | INFO 23 | DIAGNOSTIC_MODE |  |  |
|  |  | INFO 24 | SWITCH POWER OFF |  |  |
|  |  | INFO 25 | TEMP_TRAVEL |  |  |
|  |  | INFO 26 | TEMP_LIFT |  |  |
|  |  | INFO 27 | TEMP_STEER |  |  |
|  |  | INFO 28 | SPARE_1 |  |  |
|  |  | INFO 29 | DISPLAY SWITCHES THE EXTERNAL BUZZER |  |  |

|  |  | INFO 30 | TWIN PEDAL BOTH APPLIED |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | INFO 31 | NO ZERO POSITION |  |  |
|  |  | INFO 32 | NO TRAVERSE REFERENCE ON POWER UP |  |  |
|  |  | INFO 33 | ONLY CRAWL SPEED POSSIBLE AS BATTERY NOT LOCKED |  |  |
|  |  | INFO 34 | ONLY CRAWL SPEED POSSIBLE, AS INFO FROM STEERING CONTROLLER |  |  |
|  |  | INFO 35 | LOW VOLTAGE TRAVEL |  |  |
|  |  | INFO 36 | LOW VOLTAGE LIFT |  |  |
|  |  | INFO 37 | LOW VOLTAGE STEER |  |  |
|  |  | INFO 40 | CHECK MOTOR SENSOR SYSTEM |  |  |
|  |  | INFO 48 | IMPACT EVENT VERTICAL FORCE 1 |  |  |
|  |  | INFO 49 | IMPACT EVENT VERTICAL FORCE 2 |  |  |
|  |  | INFO 50 | IMPACT EVENT VERTICAL FORCE 3 |  |  |
|  |  | INFO 51 | IMPACT EVENT HORIZONTAL FORCE 1 |  |  |
|  |  | INFO 52 | IMPACT EVENT HORIZONTAL FORCE 2 |  |  |
|  |  | INFO 53 | IMPACT EVENT HORIZONTAL FORCE 3 |  |  |
|  |  | INFO 54 | SET TRUCK TYPE |  |  |
|  |  | INFO 73 | CHECK VALVE OUTPUTS |  |  |